24 Fall ECEN 704: VLSI Circuit Design

Design Pre-lab Report

Lab6: Differential Pairs

Name: Yu-Hao Chen

UIN:435009528

Section:601

Professor: Aydin Karsilayan

TA: Troy Buhr

- Compare the two single-ended differential amplifiers discussed in this lab. Rate the differential gain, common-mode gain, output voltage swing range, CMRR and common-mode input voltage range. Include expressions for each design specification.

- Design the simple differential amplifier in Figure 6-6 to obtain the following specifications:
Slew Rate > 10 V/μs
Gain-Bandwidth Product > 5 MHz
Common-mode Input Voltage Range > 0.5 V
Power Supply VDD = -VSS = 0.9 V
Load Capacitance 10 pF

- Design the differential amplifier with cascode current mirrors in Figure 6-8 to obtain the following
specifications:
Slew Rate > 10 V/μs
Gain-Bandwidth Product > 5 MHz
CMRR > 60 dB
Power Supply VDD = -VSS = 0.9 V
Load Capacitance